ON-VEHICLE SERVICE

CUSTOMIZATION FUNCTION

Using the ETACS system of scan tool MB991958,

the following functions can be programmed. The pro- grammed information is held even when the battery

is disconnected.

M1545002501378

| Adjustment item (scan tool MB991958 display) | Adjustment item | Adjusting contents (scan tool MB991958 display) | Adjusting contents |
| --- | --- | --- | --- |
| ACC power auto cut | Time to ACC power cut-off when the ignition switch is in the ACC position | Disable | No function |
|  |  | 30 min | 30 minutes (initial condition) |
|  |  | 60 min | 60 minutes |
| Turn power source | Adjustment of turn-signal light operation condition | ACC or IG1 | Operable with ACC or ON position |
|  |  | IG1 | Operable with ON position (initial condition) |
| Comfort flasher | With/without comfort flasher function | Disable | Without function |
|  |  | Enable | With function (initial condition) |
| Comfort flasher switch time | Switch operation time to activate the comfort flasher function | Normal | 0.4 second (initial condition) |
|  |  | Long | 0.8 second |
| Hazard answer back | Adjustment of the number of keyless hazard warning light answer back flashes | Lock:1, Unlock:2 | LOCK: Flashes once, UNLOCK: Flashes twice (initial condition) |
|  |  | Lock:1, Unlock:0 | LOCK: Flashes once, UNLOCK: No flash |
|  |  | Lock:0, Unlock:2 | LOCK: No flash, UNLOCK: Flash twice |
|  |  | Lock:2, Unlock:1 | LOCK: Flash twice, UNLOCK: Flash once |
|  |  | Lock:2, Unlock:0 | LOCK: Flash twice, UNLOCK: No flash |
|  |  | Lock:0, Unlock:1 | LOCK: No flash, UNLOCK: Flash once |
|  |  | Lock:0, Unlock:0 | Without function |

| Adjustment item (scan tool MB991958 display) | Adjustment item | Adjusting contents (scan tool MB991958 display) | Adjusting contents |
| --- | --- | --- | --- |
| Front wiper operation | Adjustment of the intermittent windshield wiper operation <vehicles without auto light> | Normal INT | Intermittent wiper interval is fixed to 4 seconds. |
|  |  | Variable INT | Intermittent wiper interval is calculated only by the wiper volume control. |
|  |  | Speed Sensitive | Intermittent wiper interval is calculated according to the intermittent wiper volume control and vehicle speed (initial condition). |
|  | Adjustment of the intermittent windshield wiper operation <vehicles with auto light> | Normal INT | Intermittent wiper interval is fixed to 4 seconds. |
|  |  | Variable INT | Intermittent wiper interval is calculated only by the wiper volume control. |
|  |  | Speed Sensitive | Intermittent wiper interval is calculated according to the intermittent wiper volume control and vehicle speed. |
|  |  | Rain Sensitive | Intermittent wiper interval is calculated according to the intermittent wiper volume control and lighting control sensor (initial condition). |
| Front wiper washer | Disabling or enabling washer-linked wiper function | Only Washer | No function |
|  |  | Washer & Wiper | With function: Without delayed finishing wipe function (Initial condition) |
|  |  | With after wipe | With function: With delayed finishing wipe function |
| Intelligent washer | With/without intelligent washer function | Disable | No function |
|  |  | Enable | With function (initial condition) |
| Intermittent time of rear wiper | Adjustment of rear wiper interval | 0 sec | No wiper interval |
|  |  | 4 sec | 4 seconds |
|  |  | 8 sec | 8 seconds (initial condition) |
|  |  | 16 sec | 16 seconds |
| Rear wiper low speed mode | Disabling or enabling rear wiper continuous operation | Disabled | Without function |
|  |  | Enabled | With function (initial condition) |
| Rear wiper (linked activated when in reverse | Adjustment of automatic rear window wiper operation with reverse gear engaged | Enable(R wip.ON) | Operates only when the rear wiper switch is ON. |
|  |  | Enable(R/F wip.) | Operates only when the front or rear wiper switch is ON (initial condition). |

| Adjustment item (scan tool MB991958 display) | Adjustment item | Adjusting contents (scan tool MB991958 display) | Adjusting contents |
| --- | --- | --- | --- |
| Sensitivity for auto light | Lighting control sensor sensitivity (illumination intensity) <vehicles with auto light> | Level 1 bright | High-high ambient brightness |
|  |  | Level 2 bright | High ambient brightness |
|  |  | Level 3 | Standard ambient brightness (initial condition) |
|  |  | Level 4 dark | Low ambient brightness |
|  |  | Level 5 dark | Low-low ambient brightness |
| Dome light delay timer with door | Adjustment of interior light delay shutdown time | 0 sec | 0 second (no delay shutdown time) |
|  |  | 7.5 sec | 7.5 seconds |
|  |  | 15 sec | 15 seconds |
|  |  | 30 sec | 30 seconds (initial condition) |
|  |  | 60 sec | 60 seconds |
|  |  | 120 sec | 120 seconds |
|  |  | 180 sec | 180 seconds |
| Headlight auto cut customize | Adjustment of headlight automatic shutdown function | Disable | Without function |
|  |  | Enable (C-spec.). | With function (initial condition) |
| Welcome light | Disabling or enabling welcome light function | Disabled | No function |
|  |  | Small light | Tail light illuminates. (initial condition) |
|  |  | Head light | Headlight illuminates. |
| Coming home light | Disabling or enabling coming home light function | Disabled | No function |
|  |  | 15 sec | Headlight illuminates for 15 seconds. |
|  |  | 30sec | Headlight illuminates for 30 seconds. (initial condition) |
|  |  | 60 sec | Headlight illuminates for 60 seconds. |
|  |  | 180 sec | Headlight illuminates for 180 seconds. |
| Interior light auto cut timer | Adjustment of interior light automatic shutdown function operation time | Disabled | Without function |
|  |  | 3 min | 3 minutes |
|  |  | 30 min | 30 minutes (initial condition) |
|  |  | 60 min | 60 minutes |
| Door unlock mode | Adjustment of power door locks with selective unlocking | All doors unlock | Without function: The first operation of keyless entry system or unlock operation by KOS unlocks all doors. |
|  |  | Dr door unlock | With function: The first operation of keyless entry system or unlock operation by KOS unlocks the driver's door only, and the second unlock operation within 2 seconds after that unlocks all doors. (initial condition) |

| Adjustment item (scan tool MB991958 display) | Adjustment item | Adjusting contents (scan tool MB991958 display) | Adjusting contents |
| --- | --- | --- | --- |
| Auto door unlock | Adjustment of the auto door unlock function | Disabled | Without function (initial condition) |
|  |  | Always (P pos) | With function: Operates when the shift lever or the selector lever is moved to the P position. |
|  |  | Always(Lock pos) | With function: Operates when the ignition switch is moved to the LOCK (OFF) position. |
| Duration of horn chirp | Horn sounding time during horn answer back | Short | 0.01 second (initial condition) |
|  |  | Long | 0.02 second |
| Sensitivity for theft sensor | Sensitivity level setting of theft-alarm sensor <Vehicles with theft-alarm sensor> | Level 1 | 100% sensitivity of theft-alarm sensor (initial condition) |
|  |  | Level 2 | 90% sensitivity of theft-alarm sensor |
|  |  | Level 3 | 80% sensitivity of theft-alarm sensor |
|  |  | Level 4 | 70% sensitivity of theft-alarm sensor |
| Horn chirp by keyless | Horn chirp by keyless entry system <vehicles without auto light> | Not sound horn | No horn answerback function |
|  |  | Lock any time | The horn sounds when the lock button of keyless entry transmitter is pressed once. |
|  |  | W lock any time | The horn sounds when the lock button of keyless entry transmitter is pressed twice. (initial condition) |
|  | Horn chirp by keyless entry system <vehicles with auto light> | Not sound horn | No horn answerback function |
|  |  | Lock any time | The horn sounds when the lock button of keyless entry transmitter is pressed once. |
|  |  | Lock/auto ON | During daytime, while the lighting switch is in the AUTO position, the horn sounds once when the lock is pressed once. |
|  |  | W lock any time | The horn sounds when the lock button of keyless entry transmitter is pressed twice. (initial condition) |
| Buzzer answer back | Adjusts the tone alarm answer back function. | Not sound tone alarm | Without function |
|  |  | At keyless | Sounds when the keyless entry system is activated. |
|  |  | At F.A.S.T.* | Sounds when KOS is activated (initial condition). |
|  |  | At Both | Sounds when the keyless entry system or KOS is activated. |
| Timer lock timer | Timer lock period adjustment | 30 sec | 30 seconds (initial condition) |
|  |  | 60 sec | 60 seconds |
|  |  | 120 sec | 120 seconds |
|  |  | 180 sec | 180 seconds |

| Adjustment item (scan tool MB991958 display) | Adjustment item | Adjusting contents (scan tool MB991958 display) | Adjusting contents |
| --- | --- | --- | --- |
| Panic alarm switch | With/without panic alarm function | Disable | Without function |
|  |  | Enable | With function (initial condition) |
| F.A.S.T.* key detect out fm window | With/without KOS key exterior detection function | Enable | Without function |
|  |  | Disable | With function (initial condition) |
| F.A.S.T.* feature | KOS function adjustment | Both enable | All KOS functions are enabled (initial condition). |
|  |  | DoorEntry enable | Only door entry function is enabled. |
|  |  | ENG strt enable | Only engine starting function is enabled. |
|  |  | Both disabled | All KOS functions are disabled. |
| F.A.S.T.* unlock disable time | Adjusts the door unlock inhibition period after door lock is activated. | 0 sec | 0 second |
|  |  | 3 sec | 3 seconds (initial condition) |
|  |  | 5 sec | 5 seconds |

NOTE: *: F.A.S.T. (Free-hand Advanced Security

Transmitter)